A330 Panel “Instruction Manual”

Version 1.4.0-FMC for ProFlight 98   (10th February 1999)

To get the best out of this FS98 panel it is essential you read this file in full.

This is a highly complex panel and in fact one of the most complicated panels available for FS98. Many of the functions will be missed completely without a full understanding of the contents of this manual. I know it’s a drag reading yards of text, but to get the full enjoyment from this panel it is really necessary.

**************************************************************************************

Introduction

This panel was Designed and Developed by Roy Chaffin      (roy@roychaffin.com)

For the Airbus 330

The panel is dedicated to my good friend Alan Landsburgh, in appreciation for supplying the brilliant cockpit photo that enabled this panel to be better than 95% accurate. Plus his help in interpretation, and acting as a “sounding board” for my ideas.

To date this is by far the most advanced panel ever created for the Airbus 330 family and brings a realism never before seen on an A330  panel. It has been created with the co-operation of the best names in the game.

It is for use with FS98 in Voodoo 2 mode and all other known 3D modes. This panel is designed to be used at 1024 x 768 and above. It will not function properly at lower resolutions.

It is also NOT recommended for 2D graphics cards, although it might work, the “frame rate” will probably be unbearable.

The panel is fairly graphic intensive and is only for top end PC's. Slower machines will work but with very slow frame rates. It was built and tested on a PII 450mhz with Creative RIVA TNT video card and twin Voodoo II cards.

The Microsoft FS98 Converter is **ESSENTIAL**

ProFlight 98 is **ESSENTIAL**

The panel is setup to display the SHAREWARE PFD & ND gauges which show this panel off to its best. These gauges will malfunction on each flight after 10 minutes unless registered. However if you prefer there are alternative, less accurate FREEWARE gauges. These have already been automatically installed and can be invoked by replacing the "Panel.cfg" file in the Panel Sub folder with the one contained within the "Freeware-Config" Zip file in the "Backup Files” folder. Should you wish to change back to the "SHAREWARE" gauges at any time just replace the Panel.cfg file in the Panel sub folder with the one contained within the " Shareware-Config" Zip file in the "Backup Files” folder.

Virtually all the gauges have been specially designed or massively modified to look and work as closely as possible to the full sized aircraft. Some changes are essential to enable you to fly it successfully with FS98.

Credits

I would like to thank Eric Ernst, for some of his gauges which I modified for this panel.

I wish to sincerely thank my good friend Chuck Dome for the hours of work he has committed to this project, enabling us to create some stunning new gauges. In particular the EICAS gauge. From my graphics he has created a gauge that looks and works exactly as the full size aircraft. I am also really pleased with the "Moving Trim Wheel and Throttle" gauge that we have created together. From a semi flippant idea I floated past Chuck, his brilliant programming skills with my minor contribution of the graphics, have created another fabulous gauge.

I also wish to deeply thank Tony D'AMBROSIO, who’s work on this project has been tireless. He has programmed a wonderfully accurate FREEWARE Autopilot from my graphics, which when used in conjunction with his A330 PFD and ND gauges give this panel a truly realistic feel, second to none. Also for the FREEWARE gauges used in the "Upper Panel", and the Radio Stack.

Without the enthusiastic involvement of these "experts" this accurate panel would still be a dream.

This panel is FREEWARE for your enjoyment, however to get the full benefit and accuracy, it is strongly recommended that you register Tony D'AMBROSIO's SHAREWARE PFD and ND gauges. This now includes a “Map Mode” display making the Autopilot nearly fully functional.

Thanks also go to William S. Grabowski for his invaluable help in teaching me about 3D bitmaps.

Also “special thanks" to:-

The long suffering Beta Testers.

To Microsoft for FS98 and the SDK

Lastly, to anyone I might have forgotten.

Legal

Owing to the continued abuse of International Copyright Law from a few Freeware and Payware developers, no one is permitted to use any part of this package either "as is" or "modified" unless specific prior permission is given by Roy Chaffin. Requests for permission are to be sent direct to "roy@roychaffin.com" before any attempt is made to use any part of this package, however small.

A "CREDIT" in your documentation is NOT sufficient.

Any abuses will be dealt with in a very harsh manner. YOU HAVE BEEN WARNED

It is forbidden to make money with this panel, or any of its components.

It must not be included in any software collection, BBS or CS Forum where any payment is expected or

received.

# Software Requirements

To use this panel you NEED:

1. Microsoft Flight Simulator 98 For Windows 95

2. Microsoft FS98 Converter / Upgrade installed.

3. 3DFX / Voodoo II Board or other 3D card is required for best performance.

4. WinZip v6.3 or above

5. A fairly fast PC is necessary for best performance.

6. ProFlight 98

7. ND99_10.zip or higher must be downloaded and installed, for Map Mode to function.

Installation:

The Zip files contain the Panel, and the Gauges

Unzip A330-FMC.zip to a Temporary folder, then proceed as follows:

Copy the "A330-RCS-FMC " folder into your \aircraft\ProFlt98 folder of your Flight Simulator.

Copy the files contained in the "Gauges" folder into the Flight Simulator "Gauges" subfolder.

Assign the panel to an aircraft in the usual manner.

*********WARNING***********

You MUST NOT set this aircraft/panel as your DEFAULT. This will cause FS98 to CRASH because of the heavy demands on your system.

You should always "COLD REBOOT" your system before starting FS98, and close down any unnecessary programs including Virus Checkers.

Changing resolutions while this aircraft/panel is selected is likely to cause a system CRASH.

THESE ARE NOT FAULTS OF THE AIRCRAFT OR PANEL, BUT LIMITATIONS OF FS98

*********WARNING***********

Quick Notes about this Panel

Note 1:

To toggle "On and Off" the "Throttles Overlay" click your left hand mouse button on the "Large Rivet" to the right of and below the middle MFD.

Note 2:

The FMC can be seen by clicking the "FMC" button to the top left of the left-hand MFD.

See “Instructions.jpg” (on the Instructions picture the button is marked “GPS” on the panel it is “FMC”

Note 3:

There are no "Seat Belts & No Smoking" signs in this version.

Note 4:

It is essential that the aircraft used with this panel, has had the aircraft model especially modified, so that there are only 4 positions for the Flaps/Slats. If you need to use ADE98 to make the modifications to the .sim file in the aircraft folder.

Take-off flap setting is "Position 2"

Note 5:

DO NOT use "SHFT 1 - 9" to change views on the panel. This just "cocks it up".

Note 6:

This version does contain the "Moving Map display" ND99_10.zip must be downloaded and installed for this feature to function.

Note 7:

Have a look at the "Instructions,jpg" file to find the various buttons and switches.

Note 8:

When operating the "AUTO THROTTLE" both the AUTO THROTTLE button and the SPD / MACH buttons must be depressed (with yellow lights on) for it to work.

Note 9:

Read the “Detailed Notes about this Panel” completely, once you have had a quick “Play”

Detailed Notes about this Panel

AutoPilot

The A330 Autopilot is as close to the operation of the real thing as it is possible to be within the constraints of FS98. Some changes have been essential to make it function properly. Some knobs and switches do not function in this version, See “Instructions.jpg” for details.

Engine Starter and Fuel Switches

The Engine Starters and Fuel Selectors switches in the upper panel are all fully operational.

Throttle Levers

The Throttle Levers are fully functional and mouse controllable.

Radio Stack Panel

The Radio Stack panel is fully operational and setting changeable with the mouse.

“Arming” the Speedbrakes

You can “ARM” the speedbrakes for automatic deployment on touchdown, by Left mouse clicking just below the “1/4 speedbrake” mark.

The knob will drop below the 1/4 position indicator.

Flaps & Slats

Unlike most commercial aircraft the A330 has four programmed positions for the Flaps/Slats in the Manual Mode. UP, 2, 3, FULL.

Although on the real A330 in Auto Mode it is infinitely variable, that is not possible with FS98.

The Flap/Slat display on the EICAS MFD gives a good graphical view of how much flap you have deployed.

Moving Map Display

This facility is now fully operational. However you need to download ND99_10.zip or higher for this to be fully functional. More modes are to be added to this gauge in the future.

Moving Trim Wheels

The "Moving Trim Wheels" are particularly visually effective when viewed with the "Autopilot Altitude"

function engaged; the wheels will rotate and the scale move as you either manually change the Trim or with the Autopilot. Manual change of the trim can be effected by use of the "HOME" and "END" keys on your keyboard.

PFD and ND Shareware Gauges

To get the full "Realism" of this panel it is necessary to use Tony's two brilliant PFD and ND  SHAREWARE gauges. Although Shareware gauges are not the norm in the Flight Sim community, when the gauges are so good and so different from anything else available, they deserve support.

Tony D'AMBROSIO has spent many hours producing superb FREEWARE gauges for this project. I strongly recommend that you repay these many hours of effort by registering the two SHAREWARE gauges at the very reasonable price. This will also enable you to use all current versions of the PFD and ND gauges. The ND gauges now include Map Mode.

To Register the two gauges, read and complete the “Register.txt” included with this package.

ARC / ROSE / MAP / PLAN Displays on the ND Gauge

You can change from "ARC", to "ROSE" , to “MAP” or to “PLAN” display on the ND (Middle MFD) by clicking on the Left hand rotary switch on the Autopilot (there are several setting for each mode). This switch is now fully operational. You can also change  the "Range" of the MAP / Plan with the rotary switch that is second from the left.

PFD Switch

When using Tony's Shareware PFD, you can switch the FD from "OFF" to "ALT/HDG" or "NAV" by use of the switch to the upper left of the left-hand MFD. Just Left mouse click on the switch. This switch does not function in the full Freeware version. (See “Instructions.jpg)”

Parking Brakes

The parking brakes can be toggled "ON" or "OFF" by clicking on the handle with the mouse.

Master Warning, Stall, and OverSpeed Lights

There is a "Master Warning Light" on the far left of the panel containing the Autopilot and "Stall" and "OverSpeed" warning lights to the right of the master warning light. (See the Instructions.jpg file)

Autobrake System

The original gauge was written by Laurent Crenier, and his basic "engine" behind this gauge remains the same. However all the graphics in the gauge have been rewritten for the A330 by Roy Chaffin.

INTRODUCTION

This gauge for Microsoft Flight Simulator 98 is the first attempt to simulate the complex autobrake system of a big jet. When armed, it operates in conjunction with the automated flight equipment to pilot the landing gear primary brake pressurization system to generate predictable deceleration during a Rejected Takeoff (RTO) or during landing. The rate of deceleration can be preset by a cockpit selector, which is the "visible part" of the gauge.

INSTRUCTIONS FOR USE        -         SEE NEXT PAGE

INSTRUCTIONS FOR USE

To use the Autobrake guage, set the gauge by simply ‘left mouse clicking’ on the left or right Autobrake lights to change settings as specified below.

The positions are :

RTO	TWO RIGHT HAND RED LIGHTS

Left click mouse on Lefthand "Autobrake Light Window" until two red lights appear in

"Righthand Autobrake Light Window"

If selected, provides MAX braking pressure automatically if the throttles are moved to

idle after reached 85 knots during takeoff roll  (It is the Rejected Takeoff situation).

RTO selection is not permitted in flight.

From the “RTO” position you can toggle through the different settings by Left clicking the mouse on Righthand "Autobrake Light Window" until the desired setting is reached. You can toggle back through the setting by Left clicking on the Lefthand “Autobrake Light Window”.

OFF	NO LIGHTS IN AUTOBRAKE DISPLAY

Nothing happens ! Autobrake system is OFF. If you switch to OFF when autobrake is

active, braking stops immediately.

DISARM	ONE YELLOW LIGHT IN EACH AUTOBRAKE LIGHT WINDOW

The selector goes automatically to this position when the autobrake system disarms itself.

When selected manually, is similar to OFF.

Level 1	TWO ORANGE LIGHTS IN LEFTHAND AUTOBRAKE LIGHT WINDOW

Provides LOW deceleration rate during a normal landing (see Technical Data).

Level 2	TWO ORANGE LIGHTS IN MIDDLE AUTOBRAKE LIGHT WINDOW

Provides MEDIUM deceleration rate during a normal landing (see Technical Data).

Level 3	TWO BLUE LIGHTS IN LEFTHAND AUTOBRAKE LIGHT WINDOW

Provides LOW deceleration rate during landing on WET runway (see Technical Data).

Level 4	TWO BLUE LIGHTS IN MIDDLE AUTOBRAKE LIGHT WINDOW

Provides MEDIUM deceleration rate during landing on WET runway

(see Technical Data).

MAX AUTO	TWO ORANGE LIGHTS IN RIGHTHAND AUTOBRAKE LIGHT WINDOW

Provides MAXIMUM braking pressure during normal landing on short runways

(see Technical Data).

VERY DETAILED INFORMATION ON AUTOBRAKE SYSTEM

AUTOBRAKE LOGIC

The autobrake computer which pilots the system is active when the selector is on any position except OFF and DISARM. It detects automatically the ground, takeoff and landing phases of the flight. The autobrake actions are determined accordingly. The TAKEOFF phase begins when 85 knots are reached on the runway and ends when the plane leaves the ground.

The LANDING phase begins at touchdown and ends when the ground speed is less than 10 knots.

The autobrake system will automatically apply maximum brake pressure during the TAKEOFF phase (that is when speed is greater than 85 knots) if RTO is selected AND throttles are moved to IDLE (Rejected Takeoff situation).

The system will automatically disarm when the target ground speed is reached (<10 knots) OR if throttles are moved forward. Manual braking doesn’t disarm autobrake during RTO automatic braking.

The autobrake system will automatically apply appropriate brake pressure during the LANDING phase if levels 1 to 4 or MAX AUTO are selected (see Technical data). The system will automatically disarm when the target ground speed is reached (<10 knots) OR if throttles are moved forward (Go/Around situation) OR in case of manual braking. Nevertheless, manual braking doesn’t disarm autobrake during MAX AUTO automatic braking.

The autobrake system will also automatically disarm in the following situations :

a. On the ground, during the TAKEOFF phase, when RTO is selected and brakes are applied manually.

b. During normal takeoff with RTO selected, autobrake switches to OFF just after the liftoff.

c. If throttles are advanced beyond idle after touchdown, with levels 1 to 4 or MAX AUTO selected

(Go/Around situation).

d. When the target speed (<10 knots) is reached by the automatic braking.

TECHNICAL DATA

This gauge has been tested during "standardised" landings (ILS approach),  with the FS98 Boeing 737-400 at the Brussels International Airport (EBBR) on the runway 25L, without winds.

You find here the length (in seconds) to decelerate from touchdown to less than 10 knots (the moment when the autobrake switches to disarm) with each autobrake status. Speed at touchdown was 140 knots, the spoilers were not extended and the thrust reversers not used.

Autobrake Status			Time to disarm

______________________________________________________

MAX AUTO .............................	14 seconds

LEVEL 4 ..............................	17 seconds

LEVEL 3 ..............................	35 seconds

LEVEL 2 ..............................	47 seconds

LEVEL 1 ..............................	65 seconds

Registration

You do not need to register it, this software FREEWARE. (with the exception of Tony's PFD and ND)

If you like it use it, if not, delete it.

I accept absolutely no responsibility for YOUR use of this software. There is no reason that it should in anyway damage your machine, but if it does, hard luck. If you don't accept this risk don't use it.

If you wish to use any gauges either as is, or modified, or any part of this panel in any panel you wish to publish even as as FREEWARE you are requested to contact ME out of Courtesy before doing so, at "roy@roychaffin.com"

General

This panel is the result of five months work by a flight simulator enthusiast to increase the enjoyment of all those people who partake in the hobby. Hopefully it might inspire other designers to greater and better things.

If you like it, USE it and tell me by email, if not DUMP it, but do tell me what you didn’t like so improvements can be made in future panels.

Please remember the only thing FREEWARE developers get for all their hard work is your comments.

So don't be shy.

Happy flights, and safe landings.

Roy Chaffin "roy@roychaffin.com"